🔌 2. FULL CONNECTION (IMPORTANT)

🔵 HC-05 Bluetooth → Arduino Uno

⚠️ DO NOT connect TX-RX directly wrong

| HC-05 Pin | Arduino Pin |
| --- | --- |
| VCC | 5V |
| GND | GND |
| TXD | Pin 10 |
| RXD | Pin 11 (through voltage divider ⚠️) |

⚠️ VERY IMPORTANT (RX protection)

Arduino TX (5V) → HC-05 RX (3.3V safe)

Make voltage divider:

- 1K resistor from Arduino pin 11 → HC-05 RX

- 2K resistor from HC-05 RX → GND

⚙️ L298N Motor Driver → Arduino

| L298N Pin | Arduino Pin |
| --- | --- |
| IN1 | 4 |
| IN2 | 3 |
| IN3 | 6 |
| IN4 | 7 |
| ENA | 5 (PWM) |
| ENB | 9 (PWM) |
| GND | GND |

🔋 Motor Power (MOST IMPORTANT)

| Power | Connection |
| --- | --- |
| Battery + | L298N +12V |
| Battery - | L298N GND |
| Arduino GND | MUST connect to same GND |

⚠️ If GND not common → car will NOT work